Bedrock Aquifer Systems for the Maumee River Basin, Indiana

Metadata:

- Identification_Information

- Data_Quality Information

- Spatial_Data_Organization_Information

- Spatial_Reference_Information

- Entity_and_Attribute_Information

- Distribution_Information

- Metadata_Reference_Information

IDENTIFICATION_INFORMATION

Citation:

Citation_Information:

Originator:

Randy Maier, IDNR Division of Water, Ground Water Section

Publication_Date: 1996

Title: Maumee River Basin Bedrock Aquifer Systems

Edition: 1.0

Geospatial_Data_Presentation_Form: Map

Publication_Information:

Publication_Place: Indianapolis, Indiana, USA

Publisher: Indiana Department of Natural Resources, Division of Water

Other_Citation_Details:  plate 1 of the Maumee River Basin study. This map was compiled from multiple unpublished work maps at varying scales.  Authors included: Randy Maier and Bill Herring

Online_Linkage: http://www.ai.org/dnr/water/maumee/maumee.html

Larger_Work_Citation:

Citation_Information:

Originator: Indiana Department of Natural Resources, Division of Water

Publication_Date: 1996

Title: Water Resource Availability in the Maumee River Basin, Indiana

Publication_Information:

Publication_Place: Indianapolis, IN

Publisher: Indiana Department of Natural Resources, Division of Water

Online_Linkage: http://www.ai.org/dnr/water/maumee/maumee.html

Description:

Abstract:  This file is a digital geospatial polygon coverage of bedrock aquifer systems mapped for the Maumee River Basin in Indiana.

Purpose:   Display and/or analyses requiring spatial distribution of bedrock aquifer systems in the Maumee basin.

Supplemental_Information:

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date:

Ending_Date:

Currentness_Reference:

Status:

Progress: Complete

Maintenance_and_Update_Frequency: None planned

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: -85.3473

East_Bounding_Coordinate: -84.7636

North_Bounding_Coordinate: 41.7693

South_Bounding_Coordinate: 40.6070

Keywords:

Theme:

Theme_Keyword_Thesaurus: None

Theme_Keyword: bedrock aquifer systems

Place:

Place_Keyword_Thesaurus: None

Place_Keyword: Indiana, USA

Access_Constraints: This file is available to anyone, but access may be contingent on written request, specific terms relevant to the agency or person making the request, and/or current freedom of information statutes in the State of Indiana.

Use_Constraints:  Acknowledgement of the following agency in products derived from these data:  Indiana Department of Natural Resources, Division of Water.

This map was compiled by staff of the Indiana Department of Natural Resources, Division of Water over many years using data believed to be accurate.  However, a degree of error is inherent in all maps.  This product is distributed “as-is” without warranties of any kind, either expressed or implied.  The maps are intended for use only at the published scale.

The information on these media is proprietary to the State of Indiana, Indiana Department of Natural Resources, Division of Water.  Any copying, adaptation, or distribution of these data without the express written consent of IDNR-DOW is discouraged.

It is not recommended that these data be used for purposes other than general estimates of aquifer characteristics. It should also be noted that characteristics are variable in any given aquifer due to geologic conditions.

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Native_Data_Set_Environment:

ArcView version 3.1 shapefile format

mu_br_aq.shp

Top

DATA_QUALITY_INFORMATION

Attribute_Accuracy:

Attribute_Accuracy_Report:  The mapping was conducted after a thorough documentation of well characteristics and recording of pump test details.

Logical_Consistency_Report:

Completeness_Report: In conducting the mapping, every attempt was made to take into account all water wells that had complete information on well logs, including complete location, construction, pump test, and lithological information.  There were many areas in the basin that had insufficient information due to the particular drillers in the area who chronically under-report information on well logs or because of the pervasive insufficient supply of water.

Positional_Accuracy: The locations of the wells were field located, but the locations of actual wells were usually estimated from 7.5 minute topographic maps, and visual siting of wells was not always possible.  Many locations were based on location of the street address on the well log.

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report:

Vertical_Positional_Accuracy:

Vertical_Positional_Accuracy_Report:

Lineage:

Source_Information:

Source_Citation:

Citation_Information:

Originator:  Indiana Department of Natural Resources, Division of Water

Publication_Date: 1996

Title:  Maumee River Basin Aquifer Systems, Indiana

Edition: 1.0

Geospatial_Data_Presentation_Form: map

Publication_Information:

Publication_Place: Indianapolis, IN

Publisher: State of Indiana, IDNR

Other_Citation_Details: plate 1, Maumee River Basin Study

Online_Linkage: http://www.ai.org/dnr/water/maumee/maumee.html

Larger_Work_Citation:

Citation_Information:

Originator: Indiana Department of Natural Resources, Division of Water

Publication_Date:  1996

Title: Water Resource Availability in the Maumee River Basin, Indiana

Publication_Information:

Publication_Place: Indianapolis, IN

Publisher: Indiana Department of Natural Resources, Division of Water

Online_Linkage: http://www.ai.org/dnr/water/maumee/maumee.html

Source_Scale_Denominator:

Type_of_Source_Media: digital file

Source_Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date:

Ending_Date:

Source_Currentness_Reference: Publication date

Source_Citation_Abbreviation: IDNR_DOW_WFWR_Bedrock_Aquifer_Systems

Source_Contribution: water well logs from the IDNR-DOW well log archives; Hydrogeologic Atlas of Indiana by Geosciences Research Associates, Inc. and the Water Resources Research Center at Purdue University from 1982.  IGS hydrogeologic settings coverage; IDNR-DOW bedrock topography coverage.

Process_Step:

Process_Description: Data were selected by careful evaluation of water well logs to determine if the logs were located and if they contained complete and reasonable information.

Source_Used_Citation_Abbreviation: IDNR-DOW water well logs for domestic wells submitted by water-well drillers from 1956-1996

Process_Date: 1996

Source_Produced_Citation_Abbreviation:

Process_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description: After water well log selection was complete, other attributes were considered in the mapping process.  For example, aquifer thickness, pump rates, well depths, geology, depth to bedrock, hydrogeologic settings, ground-water availability, bedrock topography, and previously published information were all used in deciding where to map bedrock aquifer systems.

Source_Used_Citation_Abbreviation:

Process_Date: 1996

Source_Produced_Citation_Abbreviation:

Process_Contact: Judith Beaty

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description: The hand-mapped county-scale (1:48000) bedrock aquifer systems were digitized and edgematched in ESRI ArcEdit (ArcScan). (personnel:  Tim Kroeker and Sally Letsinger)  The maps were individually georeferenced using between 4 and 10 tics per map.  Projection UTM, Zone 16, NAD27.  (personnel:  Sally Letsinger)

Source_Used_Citation_Abbreviation:

Process_Date: 200010

Source_Produced_Citation_Abbreviation:

Process_Contact: Judith Beaty

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description:  After the maps were joined, a polygon coverage was created and attributed.  Attribute definitions are described above.  (personnel:  Sally Letsinger)

Source_Used_Citation_Abbreviation:

Process_Date: 200010

Source_Produced_Citation_Abbreviation:

Process_Contact: Judith Beaty

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Top

SPATIAL_DATA_ORGANIZATION_INFORMATION

Direct_Spatial_Reference_Method: Vector

Point_and_Vector_Object_Information:

SDTS_Terms_Description:

SDTS_Point_and_Vector_Object_Type: GT-polygon composed of chains

Point_and_Vector_Object_Count: 1

Top

SPATIAL_REFERENCE_INFORMATION

Horizontal_Coordinate_System_Definition:

Planar:

Grid_Coordinate_System:

Grid_Coordinate_System_Name: Universal Transverse Mercator

Universal_Transverse_Mercator:

UTM_Zone_Number:  16

Transverse_Mercator:

Scale_Factor_at_Central_Meridian: 0.999600

Longitude_of_Central_Meridian: -87.000000

Latitude_of_Projection_Origin: 0.000000

False_Easting: 500000.000000

False_Northing: 0.000000

Planar_Coordinate_Information:

Planar_Coordinate_Encoding_Method: Coordinate pair

Coordinate_Representation:

Abscissa_Resolution:

Ordinate_Resolution:

Planar_Distance_Units: Meters

Geodetic_Model:

Horizontal_Datum_Name: North American Datum of 1927

Ellipsoid_Name: Clarke 1866

Semi-major_Axis: 6378206.4000000

Denominator_of_Flattening_Ratio: 294.98

Top

ENTITY_AND_ATTRIBUTE_INFORMATION

Overview_Description:

Entity_and_Attribute_Overview:  The points in this file represent calculations of aquifer systems determined at or around water wells.   Some values are from detailed analyses, but most values were calculated from specific capacity information found on well logs for domestic water wells.  The attributes for each point include (name/input width/output width/item type):

- basin = 10 10 Character

- water management river basin name

- aquifer_type (integer value that identifies the type of aquifer) = 2 2 Integer

- integer code, see below

- aquifer_system = 50 50 Character

- IDNR Division of Water aquifer system name

- local_name = 50 50 Character

- local name, character field, filled-in if common local name exists

- thickness = 50 50 Character

- aquifer thickness ranges in feet

- yield = 50 50 Character

- yield ranges in gallons per minute

- contam_potential (contamination susceptibility) = 1 1 Character

- H=high, M=moderate, L=low

- comments = 100 100 Character

Aquifer types:

1	Outwash/Alluvium

2	Outwash/Alluvium Subsystem (thinner/less productive/more clay)

3	Outwash/Alluvium Subsystem (thicker/more productive/less clay)

4	Outwash Apron

# 5	Till

6	Till Subsystem (thinner/less productive)

7 	Till Subsystem (thicker/more productive)

8	Moraine

9	Moraine Subsystem

10	Dissected Till and Residuum

11	Lacustrine and Backwater Deposits

12	Lacustrine Plain

13	Buried Valley

14	Buried Valley Subsystem

15	Eolian deposits

16	Coastal Sands and Gravels

17	Sand and/or Gravel

18	Silt

19	Clay

20	Other glacial deposits:  kames, eskers

21	unassigned

.

.

.

50	unassigned

51	shale

52	siltstone

53	sandstone

54	limestone

55	dolostone

56	carbonates

57	shale, siltstone, limestone

58	unassigned

59	unassigned

60	Kentland anomaly

The file was created for the purpose of characterizing bedrock aquifer systems for the Water Resource Availability in the Maumee River Basin, Indiana.

Entity_and_Attribute_Detail_Citation:

Detailed Description:

Entity_Type:

Entity_Type_Label: POLYGON

Entity_Type_Definition: Polygons represent the distribution of bedrock aquifer systems that contain similar characteristics.  The source information for the mapping is retained in the database as attributes attached to the polygon.

Entity_Type_Definition_Source: IDNR-Division of Water Maumee River Basin Bedrock Aquifer Systems Project

Attribute:

Attribute_Label: basin

Attribute_Definition: water management river basin, MU = Maumee River Basin

Attribute_Definition_Source:

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

Attribute_Units_of_Measure:  string

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:

Attribute:

Attribute_Label: aquifer type

Attribute_Definition: toggle value between bedrock or bedrock

Attribute_Definition_Source:

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

Attribute_Units_of_Measure:  string

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:

Attribute:

Attribute_Label: aquifer system name

Attribute_Definition: as named by IDNR-DOW

Attribute_Definition_Source:

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

Attribute_Units_of_Measure:  string

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:

Attribute:

Attribute_Label: local name

Attribute_Definition: used if aquifer-system name does not denote a particular local name or feature.

Attribute_Definition_Source:

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

Attribute_Units_of_Measure:  string

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:

Attribute:

Attribute_Label:  Aq Thickness range

Attribute_Definition:  Thickness of the producing aquifer

Attribute_Definition_Source:  Determination by geologist using the lithologic log on the back of IDNR water well logs as well as information such as casing length and screen placement. (not usually filled in for bedrock aq syst coverage)

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 1

Range_Domain_Maximum: 500

Attribute_Units_of_Measure: feet

Attribute_Measurement_Resolution: 1 foot

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  There is a fair amount of educated guesswork in determination of aquifer thickness from a domestic water well log.  If multiple lithologies were stacked that should produce water, then the entire thickness was used.  In many cases, the well was ended in a water-bearing formation, so the thickness of the aquifer is of an assumed thickness.

Attribute:

Attribute_Label: Yield

Attribute_Definition:

Attribute_Definition_Source: IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0.1

Range_Domain_Maximum: 10,000

Attribute_Units_of_Measure: gallons per minute

Attribute_Measurement_Resolution: 0.1 gallons per minute, but usually rounded to 1 gpm

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  usually rounded up, especially on domestic water well logs in areas of low production.  Also, if the method of testing is “Air”, then the pump rate is highly inflated.

Attribute:

Attribute_Label: contamination susceptibility

Attribute_Definition: Description of the potential for aquifer contamination or transport of contaminants.

Attribute_Definition_Source:  Determined by a geologist.  Examination of recorded lithology on the water well as well as surficial geology information is used.

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution: H = high susceptibility

M = moderate susceptibility

L = low susceptibility

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  most well drillers supply enough detailed information to make this determination.

Attribute:

Attribute_Label: Comments

Attribute_Definition:  Additional information concerning the calculations.

Attribute_Definition_Source: IDNR-DOW

Attribute_Units_of_Measure: string (character)

Top

DISTRIBUTION_INFORMATION

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Resource_Description:

Distribution_Liability: The Indiana State Department of Natural Resources Division of Water has compiled this data set using data believed to be accurate; however, the Division makes no representation as to the accuracy, completeness, or usefulness of the data or data set.  Reference to any specific commercial product, process, or service does not constitute an endorsement, recommendation, or favoring by the Division.

This map was compiled by staff of the Indiana State Department of Natural Resources Division of Water using data believed to be accurate.  However, a degree of error is inherent in all maps.  This map is intended for use only at the published scale.

Copyright 1996, Indiana Department of Natural Resources

Division of Water

All Rights Reserved

Any copying, adaptation, or distribution of these data without the express written consent of the Indiana State Department of Natural Resources Division of Water is discouraged.

Top

METADATA_REFERENCE_INFORMATION

Metadata_Date: 20001025

Metadata_Review_Date:

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Judith Beaty

Contact_Position: Head, Basin Studies Section

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: jbeaty@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

Metadata_Standard_Name: FGDC CSDGM

Metadata_Standard_Version: FGDC-STD-001-1998

Metadata Use Constraints:  This metadata file is intended to accompany the subject geospatial data set.  This file may be updated, and the most recent version is available by contacting the IDNR Division of Water.

Top