Aquifer Transmissivity Values for the West Fork White River Basin

- Metadata:

- Identification_Information

- Data_Quality Information

- Spatial_Data_Organization_Information

- Spatial_Reference_Information

- Entity_and_Attribute_Information

- Distribution_Information

- Metadata_Reference_Information

- IDENTIFICATION_INFORMATION

Citation:

Citation_Information:

Originator:

- Greg Schrader, IDNR Division of Water, Basin Studies Section

Publication_Date: 200112

Title: West Fork White River Basin Aquifer Transmissivity Values, Indiana

Edition: 2.0

Geospatial_Data_Presentation_Form: Map

Publication_Information:

- Publication_Place: Indianapolis, Indiana, USA

Publisher: Indiana Department of Natural Resources, Division of Water

Other_Citation_Details:

Online_Linkage:

Larger_Work_Citation:

Citation_Information:

Originator:

Publication_Date:

Title:

Publication_Information:

Publication_Place:

Publisher:

Online_Linkage:

Description:

Abstract:  This file is a digital geospatial point coverage of transmissivity values calculated for the West Fork of the White River Basin in Indiana.  The method of calculation varies between the Jacob straight-line method for detailed analysis of pump tests and transmissivity determined from specific capacity data using the program, TGUESS.  The specific capacity data were derived from water well logs.

Purpose:   Display and/or analyses requiring spatially distributed point values of aquifer transmissivity for the West Fork White River basin.

Supplemental_Information:

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date:

Ending_Date:

Currentness_Reference:

Status:

Progress: Complete

Maintenance_and_Update_Frequency: None planned

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: -87.7633

East_Bounding_Coordinate: -84.8630

North_Bounding_Coordinate: 40.3838

South_Bounding_Coordinate: 38.3897

Keywords:

Theme:

Theme_Keyword_Thesaurus: None

Theme_Keyword: aquifer transmissivity point values

Place:

Place_Keyword_Thesaurus: None

Place_Keyword: Indiana, USA

Access_Constraints: This file is available to anyone, but access may be contingent on written request, specific terms relevant to the agency or person making the request, and/or current freedom of information statutes in the State of Indiana.

Use_Constraints:  Acknowledgement of the following agency in products derived from these data:  Indiana Department of Natural Resources, Division of Water.

This map was compiled by staff of the Indiana Department of Natural Resources, Division of Water over many years using data believed to be accurate.  However, a degree of error is inherent in all maps.  This product is distributed “as-is” without warranties of any kind, either expressed or implied.  The maps are intended for use only at the published scale.

- Copyright 2001, Indiana Department of Natural Resources

Division of Water

All Rights Reserved

The information on these media is proprietary to the State of Indiana, Indiana Department of Natural Resources, Division of Water.  Any copying, adaptation, or distribution of these data without the express written consent of IDNR-DOW is discouraged.

It is not recommended that these data be used for purposes other than general estimates of aquifer transmissivity.  Particular care should be taken with transmissivity values determined using specific capacity information from water well logs.  The user should note all accompanying that denote the quality of the calculation.  It should also be noted that transmissivity values are variable in any given aquifer due to geologic heterogeneities, so extrapolation of these values to entire aquifer systems is an incorrect use of these data.

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

Native_Data_Set_Environment:

ArcView version 3.2 shapefile format

wfwr_trans.shp

- Top

- DATA_QUALITY_INFORMATION

Attribute_Accuracy:

Attribute_Accuracy_Report:  The calculations for each point were conducted after a thorough documentation of well characteristics and recording of pump test details. The calculations using the program TGUESS from specific capacity data contain inherent assumptions, and the values for all domestic wells should be considered estimates and the data should be used with caution.

Logical_Consistency_Report:

Completeness_Report: Every attempt was made to include calculations for all water wells that had complete information on well logs, including complete location, construction, pump test, and lithological information.  There were many areas in the basin that had insufficient information due to the particular drillers in the area who chronically under-report information on well logs or because of the pervasive insufficient supply of water.  Most well logs for poorly producing wells are missing information.  A separate coverage for dry holes (or pumped/bailed dry) is included in conjunction with the transmissivity dataset.

Positional_Accuracy: The locations of the wells were field located, but the locations of actual wells were usually estimated from 7.5 minute topographic maps, and visual siting of wells was not always possible.  Many locations were based on location of the street address on the well log.

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report:

Vertical_Positional_Accuracy:

Vertical_Positional_Accuracy_Report:

Lineage:

Source_Information:

Source_Citation:

Citation_Information:

Originator:  Indiana Department of Natural Resources, Division of Water

- Publication_Date: 200112

Title:  West Fork White River Basin Aquifer Transmissivity Values, Indiana

Edition: 2.0

Geospatial_Data_Presentation_Form: map

Publication_Information:

Publication_Place: Indianapolis, IN

Publisher: State of Indiana, IDNR

Other_Citation_Details:

Online_Linkage:

Larger_Work_Citation:  Water Resource Assessment of the West Fork of the White River Basin

Citation_Information:

Originator: Indiana Department of Natural Resources, Division of Water

- Publication_Date:  not yet published; in preparation

Title: Water Resources in the White and West Fork White River Basins

Publication_Information:

- Publication_Place: Indianapolis, IN

Publisher: Indiana Department of Natural Resources

Online_Linkage:

Source_Scale_Denominator:

- Type_of_Source_Media: digital file

Source_Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date:

Ending_Date:

- Source_Currentness_Reference: Publication date

Source_Citation_Abbreviation: IDNR_DOW_WFWR_Aquifer_Transmissivity

Source_Contribution: water well logs from the IDNR-DOW well logs archives

Process_Step:

Process_Description: Data were selected by careful evaluation of water well logs to determine if the logs were located and if they contained complete and reasonable specific capacity information.

Source_Used_Citation_Abbreviation: IDNR-DOW water well logs submitted by water-well drillers from 1956-2001

- Process_Date: 1999-2001

Source_Produced_Citation_Abbreviation:

Process_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

- Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description: After data selection was complete, other attributes were added to the data set which are not explicitly stated on the water well logs.  For example, aquifer thickness was determined from the lithologic information contained on the well logs and whether the aquifer was confined or unconfined.  These additional data were needed to conduct the calculations.

Source_Used_Citation_Abbreviation:

- Process_Date: 1999-2001

Source_Produced_Citation_Abbreviation:

- Process_Contact: Greg Schrader

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

- Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description: Transmissivity calculations were conducted for water wells using specific capacity information determined from water well logs.  The computer program TGUESS was used to conduct the calculations.   The method is discussed in detail in:

Bradbury, K.R., and Rothschild, E.R., 1985, A computerized technique for estimating the hydraulic conductivity of aquifers from specific capacity data:  Ground Water, v. 23, no. 2, pp. 240-246.

Source_Used_Citation_Abbreviation: TGUESS

Process_Date: 2001

Source_Produced_Citation_Abbreviation:

Process_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

- Hours_of_Service: 8:15am - 4:15pm

Process_Step:

Process_Description:  The Jacob straight-line method was used to estimate transmissivity values for most wells that had pump tests conducted.  Unknown methods were used to determine transmissivity values for a few wells for which aquifer test reports (without data or graphs) were assembled.  These pump tests were primarily conducted for high-capacity wells (> 70 gpm).  The pump tests and calculations were conducted over many years and the calculations were conducted by many different people from within the Division of Water, industry, and consultants.  The data were gathered from the Division of Water’s county studies archive files.

- Source_Used_Citation_Abbreviation: IDNR-DOW County files

Process_Date: 1999-2001

Source_Produced_Citation_Abbreviation:

Process_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

Hours_of_Service: 8:15am - 4:15pm

- Top

- SPATIAL_DATA_ORGANIZATION_INFORMATION

- Direct_Spatial_Reference_Method: Vector

Point_and_Vector_Object_Information:

SDTS_Terms_Description:

- SDTS_Point_and_Vector_Object_Type: GT-polygon composed of chains

Point_and_Vector_Object_Count: 1

- Top

- SPATIAL_REFERENCE_INFORMATION

Horizontal_Coordinate_System_Definition:

Planar:

Grid_Coordinate_System:

- Grid_Coordinate_System_Name: Universal Transverse Mercator

Universal_Transverse_Mercator:

- UTM_Zone_Number:  16

Transverse_Mercator:

Scale_Factor_at_Central_Meridian: 0.999600

Longitude_of_Central_Meridian: -87.000000

Latitude_of_Projection_Origin: 0.000000

False_Easting: 500000.000000

False_Northing: 0.000000

Planar_Coordinate_Information:

- Planar_Coordinate_Encoding_Method: Coordinate pair

Coordinate_Representation:

Abscissa_Resolution:

Ordinate_Resolution:

- Planar_Distance_Units: Meters

Geodetic_Model:

- Horizontal_Datum_Name: North American Datum of 1927

Ellipsoid_Name: Clarke 1866

Semi-major_Axis: 6378206.4000000

Denominator_of_Flattening_Ratio: 294.98

- Top

- ENTITY_AND_ATTRIBUTE_INFORMATION

Overview_Description:

Entity_and_Attribute_Overview:  The points in this file represent calculations of aquifer transmissivity values determined at or around water wells.   Some values are from detailed analyses, but most values were calculated from specific capacity information found on well logs for water wells.  The attributes for each point include:

- reference number that corresponds to an associated water well log

- internal tracking number

- facility number

- transmissivity (1000 gallons per day per foot)

- effective well diameter (inches)

- static water level (feet)

- test level/pumping level (feet)

- drawdown (feet)

- time/length of  pump test (hours)

- pump rate (gallons per minute)

- aquifer thickness (feet)

- open interval (feet)

- storage coefficient

- setting (confined vs. unconfined aquifer)

- well loss coefficient

- adjusted specific capacity (gallons per minute per foot)

- penetration (partially or fully penetrating)

- pump flag (pumped or bailed well)

- aquifer system

- method of calculation (Jacob straight line graphical method or unknown for wells with pump tests and TGUESS for all others)

- basin (water management river basin)

- comments

The file was created for the purpose of characterizing aquifer systems for the water resource assessment of the West Fork of the White River basin.

Entity_and_Attribute_Detail_Citation:

Detailed Description:

Entity_Type:

- Entity_Type_Label: POINT

Entity_Type_Definition: Point values represent the location of water wells from which pump test information was used to calculate aquifer transmissivity.  The source information for the calculations is retained in the database as attributes attached to the point value.

Entity_Type_Definition_Source: IDNR-Division of Water WFWR Transmissivity Project

Attribute:

Attribute_Label: Ref #

Attribute_Definition: a unique reference number that corresponds to an associated water well log.

Attribute_Definition_Source: IDNR-DOW, Ground Water Section

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: -9999 (no well log available)

- Range_Domain_Maximum: 310000

Attribute_Units_of_Measure: unique value, integer

- Attribute_Measurement_Resolution: integer

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation: a unique value is assigned to each well log archived by the IDNR-Division of Water.

Attribute:

Attribute_Label:  Tracking

Attribute_Definition: Internal tracking number used when point values cannot be identified or differentiated by other means.

Attribute_Definition_Source: IDNR-DOW Basin Studies Section

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

- Attribute_Units_of_Measure: positive integers

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

- Attribute_Value_Accuracy_Explanation

Attribute:

- Attribute_Label: Facility #

Attribute_Definition: The number that is assigned to a water well (usually high capacity) by a facility, to identify the well.  Domestic wells will not have this number.

Attribute_Definition_Source:  The well owner (varies).

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:

Range_Domain_Maximum:

Attribute_Units_of_Measure:  integer or string (e.g. Well #1 or Well #3B)

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

- Attribute_Value_Accuracy_Explanation

Attribute:

Attribute_Label:  T (1000 gpd/ft)

- Attribute_Definition:  Calculated aquifer transmissivity

Attribute_Definition_Source:  Indiana Department of Natural Resources, Division of Water, Basin Studies Section

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 10,000

Attribute_Units_of_Measure: gallons per day per foot

- Attribute_Measurement_Resolution: 1000ths of gpd/ft

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation: The accuracy of the transmissivity value is dependent on the quality of the aquifer test, the recorded observations, and the validity of the assumed values used in the calculations.

Attribute:

Attribute_Label: Effective well diameter

Attribute_Definition: This value is used to represent the diameter that is used for extraction of water from the aquifer.

Attribute_Definition_Source:

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 2

Range_Domain_Maximum: 36

Attribute_Units_of_Measure: inches

Attribute_Measurement_Resolution: 0.5 inch

Attribute_Value_Accuracy_Information:

- Attribute_Value_Accuracy:  this value is an estimate

Attribute_Value_Accuracy_Explanation:  For unconsolidated wells, the effective well diameter is equivalent to the screen diameter.  It is an estimate, although most unconsolidated wells have gravel packs that would indicate that the diameter of the well is slightly larger than the screen diameter.  For bedrock wells, the diameter of the well annulus was estimated.  The nominal inside casing diameter was used unless a screen or liner was used, in which case the inside diameter of the screen or liner was used.

Attribute:

Attribute_Label:  Static level

Attribute_Definition: Static water level as recorded on the water well log; below ground level

Attribute_Definition_Source: IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: -1 (indicating flowing well, static above ground level)

- Range_Domain_Maximum: 500

Attribute_Units_of_Measure:  feet below ground surface

- Attribute_Measurement_Resolution: 1 foot

Attribute_Value_Accuracy_Information:

- Attribute_Value_Accuracy: as measured

Attribute_Value_Accuracy_Explanation:

Attribute:

- Attribute_Label: Test level

Attribute_Definition: pumping level, below ground surface

- Attribute_Definition_Source:  IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum:  0

Range_Domain_Maximum:  500

Attribute_Units_of_Measure:  feet below ground surface

- Attribute_Measurement_Resolution:  1 foot

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  calculated as static water level + drawdown

Attribute:

- Attribute_Label:  Drawdown

Attribute_Definition:  amount of drawdown measured as well was pumped

- Attribute_Definition_Source: IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 500

Attribute_Units_of_Measure: feet below ground surface

- Attribute_Measurement_Resolution: 1 foot

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

- Attribute_Value_Accuracy_Explanation

Attribute:

Attribute_Label: Time

Attribute_Definition: Length of pump test

Attribute_Definition_Source: IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0.3

Range_Domain_Maximum: 96

Attribute_Units_of_Measure: hours

Attribute_Measurement_Resolution: 0.5

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  It is likely that most drillers reported a rounded (up) estimate of the amount of time the well was pumped.  So, the Time attribute is a maximum estimate.

Attribute:

- Attribute_Label: Pump Rate

Attribute_Definition:

- Attribute_Definition_Source: IDNR-DOW water well log archives

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0.3

Range_Domain_Maximum: 1875

Attribute_Units_of_Measure: gallons per minute

Attribute_Measurement_Resolution: 0.1 gallons per minute, but usually rounded to 1 gpm

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  usually rounded up, especially on domestic water well logs in areas of low production.  Also, if the method of testing is “Air”, then the pump rate is highly inflated.  Wells were eliminated if the reported pumping rate was deemed impossibly high given the well diameter.

Attribute:

- Attribute_Label:  Aq Thickness

Attribute_Definition:  Thickness of the producing aquifer

Attribute_Definition_Source:  Determination by geologist using the lithologic log on the back of IDNR water well logs as well as information such as casing length and screen placement.

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 1

Range_Domain_Maximum: 500

Attribute_Units_of_Measure: feet

Attribute_Measurement_Resolution: 1 foot

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  There is a fair amount of educated guesswork in determination of aquifer thickness from a water well log.  If multiple lithologies were stacked that should produce water, then the entire thickness was used.  In many cases, the well was ended in a water-bearing formation, so the thickness of the aquifer is of an assumed thickness.

For bedrock and unconsolidated aquifers, if the total aquifer thickness is known, the ‘Penetration flag’ will not have an “A” (for assumed) in it.  Otherwise, the aquifer thickness is the amount of the water-bearing formation penetrated by the well (usually from the top of the formation to the bottom of the well screen).

Attribute:

- Attribute_Label: Open Interval

Attribute_Definition: This is a measure of the amount of the aquifer that is exposed to the well. For unconsolidated wells, this value is the screen length.  For bedrock wells, this value is the bedrock between the bottom of the casing and the bottom of the well (i.e., well depth – well casing length).

Attribute_Definition_Source: The determination is done by a geologist, but the values are from the IDNR water well logs.

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 1

Range_Domain_Maximum: 500

Attribute_Units_of_Measure: feet

Attribute_Measurement_Resolution: 1 foot

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  This value is based on the accuracy of information reported on the water well logs.  Furthermore, this value might be correct, but if the aquifer thickness is assumed to be equal to the thickness of aquifer penetrated, the calculated transmissivity value would represent a minimum T..

Attribute:

- Attribute_Label: Storage Coeff

Attribute_Definition: Storage coefficient.  A measure of the storage capacity of an aquifer.

Attribute_Definition_Source: Historically, the storage coefficient has been an assumed value that is dependent only on whether the aquifer is confined or unconfined.

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0.0002 (confined aquifer)

Range_Domain_Maximum: 0.2 (unconfined aquifer)

- Attribute_Units_of_Measure: unitless

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  Because this value is assumed and given hypothetical values for a given aquifer condition, the value is not particularly accurate.  The impact on the resulting calculation, however, is quite small.

Attribute:

- Attribute_Label: Setting

Attribute_Definition: Characterization of the aquifer confining condition.

Attribute_Definition_Source: Determined by a geologist.  Location of the aquifer material in the lithologic column is noted in conjunction with the static water level.

Attribute_Domain_Values:

Range_Domain:

- Range_Domain_Minimum:  C=confined

Range_Domain_Maximum: U=unconfined

- Attribute_Units_of_Measure: string

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation: The characterization of the aquifer is based on information provided on the IDNR water well logs, so the accuracy of the assessment is dependent on the accuracy of the provided information.

Attribute:

Attribute_Label:  Well Loss Coeff

Attribute_Definition: A measure of the efficiency of a well.  This value is usually larger for high-capacity wells.

Attribute_Definition_Source: Historically, this value = 1 for all wells using specific capacity data for transmissivity calculations.  This is because small diameter, low-production wells do not generate turbulence around the well screen during a pump test.

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 1

Range_Domain_Maximum: 1

Attribute_Units_of_Measure: unitless

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy: Using assumptions means that the value is not appropriate in every case.  However, we feel that the assumption is correct in most cases where it is applied.  The impact on the calculation is minor, in any case.

Attribute:

- Attribute_Label: Adjusted Specific Capacity

Attribute_Definition: Specific capacity is defined as the pump rate divided by drawdown.  Adjusted specific capacity is corrected for partial penetration of the aquifer.  For the WFWR, however, this value is merely pump rate/drawdown.

Attribute_Definition_Source: IDNR water well log data.

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 10000

Attribute_Units_of_Measure: gallons per minute per foot of drawdown

- Attribute_Measurement_Resolution: 0.01

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation: straightforward estimate; uncorrected; using well log data

Attribute:

- Attribute_Label: Penetration

Attribute_Definition: Description of the known amount of aquifer penetrated by the well.

Attribute_Definition_Source: Determined by a geologist; for unconsolidated wells, used screen length, lithology, and aquifer thickness.  For bedrock wells, used the open interval, lithology, and aquifer thickness.

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution: 	P=partially penetrating

PA=partially penetrating, aquifer thickness assumed

F=fully penetrating

FA=fully penetrating, aquifer thickness assumed

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:  The best estimates of transmissivity are records with “F” followed by “P” because the aquifer thickness is known.  Other calculations have major assumptions associated with them.

Attribute:

- Attribute_Label: Aquifer Type

Attribute_Definition: Description of the aquifer material.  For the WFWR basin, this designation is simply ‘unconsolidated’ vs. ‘bedrock.’  Other basins break the aquifer type down into lithology of the aquifer material.

Attribute_Definition_Source:  Determined by a geologist.  First breakout is by well log attributes such as ‘bedrock depth’ or presence of a screen.  Examination of recorded lithology on the water well record is ultimate source of aquifer type information.

Attribute_Units_of_Measure: string (character)

- Attribute_Measurement_Resolution: U=unconsolidated

B=bedrock

Attribute_Value_Accuracy_Information:

- Attribute_Value_Accuracy: high confidence

Attribute_Value_Accuracy_Explanation: most well drillers supply enough detailed information to make this determination.

Attribute:

Attribute_Label:  Pump Flag

Attribute_Definition: Quality flag used to differentiate between wells that have been tested by pump or bailer.

Attribute_Definition_Source:  IDNR water well logs.

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution:	P=pump test

B=bailer test (All excluded from WFWR data)

Attribute_Value_Accuracy_Information:

- Attribute_Value_Accuracy: high confidence

Attribute_Value_Accuracy_Explanation:

Attribute:

- Attribute_Label: Aquifer System

Attribute_Definition: Associated aquifer system name.  Not determined in many cases.

Attribute_Definition_Source:

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation:

Attribute:

- Attribute_Label: Method

Attribute_Definition:  Method of calculating the transmissivity value.

Attribute_Definition_Source: TGUESS program, using the method described in:

Bradbury K.R., and Rothschild, E.R., 1985, A computerized technique for estimating the hydraulic conductivity of aquifers from specific capacity data:  Ground Water, v. 23, no. 2., pp. 240-246.

The Jacob straight-line method was used for wells with structured, well designed pump tests.  The methods are described in hydrology texts such as:

Fetter, C.W., 1988, Applied Hydrogeology 2nd edition, Merrill Publishing Company, Columbus, OH, pages 164-169 (Theis method), pages 170-172 (Jacob straight-line method).

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution:

Attribute_Value_Accuracy_Information:

- Attribute_Value_Accuracy

Attribute_Value_Accuracy_Explanation: : In some cases, the exact method of calculation is not known for structured pump tests.   However, the values using any method other than TGUESS should more accurately describe transmissivity than those using specific capacity information from a domestic well log pump test.

Attribute:

- Attribute_Label: Basin

Attribute_Definition: Water management basin in which the well resides.

- Attribute_Definition_Source: IDNR well log locations

Attribute_Units_of_Measure: string (character)

Attribute_Measurement_Resolution: WFWR = West Fork White River basin

Attribute_Value_Accuracy_Information:

Attribute_Value_Accuracy:

Attribute_Value_Accuracy_Explanation: Locations were confirmed to be in the basin, or conducted for the basin report.  A few wells in counties on the boundaries of the basin were included in the analysis in case control points were needed.

Attribute:

- Attribute_Label: Comments

Attribute_Definition:  Additional information concerning the calculations.

Attribute_Definition_Source: IDNR-DOW

Attribute_Units_of_Measure: string (character)

- Top

- DISTRIBUTION_INFORMATION

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

- Hours_of_Service: 8:15am - 4:15pm

Resource_Description:

Distribution_Liability: The Indiana State Department of Natural Resources Division of Water has compiled this data set using data believed to be accurate; however, the Division makes no representation as to the accuracy, completeness, or usefulness of the data or data set.  Reference to any specific commercial product, process, or service does not constitute an endorsement, recommendation, or favoring by the Division.

This map was compiled by staff of the Indiana State Department of Natural Resources Division of Water using data believed to be accurate.  However, a degree of error is inherent in all maps.  This map is intended for use only at the published scale.

- Copyright 2001, Indiana Department of Natural Resources

Division of Water

All Rights Reserved

Any copying, adaptation, or distribution of these data without the express written consent of the Indiana State Department of Natural Resources Division of Water is discouraged.

- Top

- METADATA_REFERENCE_INFORMATION

- Metadata_Date: 200112

Metadata_Review_Date:

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Indiana Department of Natural Resources, Division of Water

Contact_Person: Greg Schrader

Contact_Position: Engineering Geologist

Contact_Address:

Address_Type: Mailing and physical address

Address: 402 W. Washington Street W264

City: Indianapolis

State_or_Province:  Indiana

Postal_Code: 46204

Country: USA

Contact_Voice_Telephone: 317-232-4160

Contact_Facsimile_Telephone: 317-233-4579

Contact_Electronic_Mail_Address: gschrader@dnr.state.in.us

- Hours_of_Service: 8:15am - 4:15pm

Metadata_Standard_Name: FGDC CSDGM

Metadata_Standard_Version: FGDC-STD-001-1998

Metadata Use Constraints: This metadata file is intended to accompany the subject geospatial data set.  This file may be updated, and the most recent version is available by contacting the IDNR Division of Water.

- Top